PROJECT

LULLABY

Product Requirements Document

Lucid Dream Induction via Consumer Wearable Technology

Version 1.0  •  February 2026

CONFIDENTIAL

# 1. Executive Summary

Project Lullaby is a lucid dream induction system built on the Apple ecosystem. It uses the Apple Watch and iPhone as a sensor fusion platform to detect when a user enters REM sleep in near-real-time, then delivers precisely timed haptic stimuli through the watch to promote lucid dreaming.

The system combines cardiac data and motion sensing from the Apple Watch with ultrasonic sonar and audio analysis from the iPhone, streams features to a cloud inference server for sleep stage classification, and calibrates itself nightly against Apple’s own sleep stage data from HealthKit. No additional hardware is required beyond what users already own.

Core Thesis If we can reliably detect REM sleep using consumer Apple hardware, and deliver a calibrated haptic cue during established REM periods, we can significantly increase the probability of lucid dream onset for trained users.

# 2. Problem Statement

Lucid dreaming — the state of being aware that one is dreaming while still asleep — has documented benefits for creativity, nightmare therapy, motor skill rehearsal, and psychological wellbeing. Research by Stephen LaBerge at Stanford demonstrated that external sensory cues delivered during REM sleep can promote lucid dream onset.

However, no commercially successful product has solved this problem. Existing approaches fall into two categories, both with critical failures:

### 2.1 Existing Solutions and Their Failures

| Approach | Products | Why It Fails |
| --- | --- | --- |
| Timer-based LED masks | Remee | No actual REM detection. Cues fire on a fixed schedule, often hitting deep sleep and waking the user. |
| EEG headbands | Neuroon, iBand+, Lucid Dreamer | Uncomfortable to wear during sleep. High abandonment rates. Most failed commercially. |

The core problem is the same in both cases: reliable REM detection from consumer hardware has been too difficult, leading to poorly timed cues that disrupt sleep rather than induce lucidity.

### 2.2 The Opportunity

Project Lullaby has a fundamental advantage over all prior attempts: people already sleep wearing Apple Watches, and they already place their iPhones on nightstands. The hardware is deployed. The sensors are present. The missing piece is the software intelligence to interpret those sensors in real-time and act on them.

# 3. Target User

The primary user is someone who is actively interested in lucid dreaming and already owns an Apple Watch (Series 4 or later, which supports sleep stage tracking) and an iPhone. They are motivated enough to wear the watch to bed, place their phone on the nightstand, and engage in pre-sleep conditioning exercises.

Primary persona: Experienced lucid dreaming enthusiast who has tried techniques like reality checks and MILD/WILD methods with inconsistent results, and wants technology-assisted induction.

Secondary persona: Curious newcomer who has heard about lucid dreaming and wants an accessible entry point with guided onboarding.

Tertiary persona: Sleep quality optimizer who is primarily interested in the detailed sleep analytics the system produces, with lucid dreaming as a bonus feature.

# 4. System Architecture Overview

The system operates across three tiers: the Apple Watch (sensor + actuator), the iPhone (sensor + relay), and a cloud server (inference + calibration). Each tier has a specific role optimized for its hardware capabilities.

## 4.1 Apple Watch (watchOS App)

The watch serves as both the primary biometric sensor and the cue delivery actuator. It is the only component physically attached to the user.

Data Collected:

- Heart rate (via optical heart sensor, sampled every few seconds during active HKWorkoutSession)

- Heart rate variability (derived from inter-beat intervals)

- Accelerometer data (via CoreMotion, up to 100Hz via CMMotionManager)

- Historical accelerometer buffer (via CMSensorRecorder, 36-hour buffer, 3-second delay)

- Gyroscope and device motion (attitude, rotation rate, user acceleration)

Actions Performed:

- Streams summarized sensor packets to iPhone via WatchConnectivity

- Delivers haptic cues at specified intensity and pattern when commanded

- Reports post-cue sensor data for wake detection

- Maintains a lightweight local fallback classifier for offline operation

## 4.2 iPhone (iOS App)

The iPhone serves as a secondary sensor platform, communication relay, and local processing hub. It sits on the nightstand, plugged in and charging.

Data Collected:

- Ultrasonic sonar: Speaker emits inaudible ~19kHz tone, microphone captures reflections off the user’s chest. Amplitude and phase modulation of the reflected signal tracks breathing rate and regularity.

- Passive audio analysis: Microphone captures ambient bedroom audio. On-device ML extracts breathing rate, breathing regularity score, snore detection and classification, body movement sounds, and sleep talking detection. Raw audio never leaves the device.

- Ambient light sensor: Detects lights-off events, confirming sleep intent.

- Device state signals: Screen lock time, charging state, last interaction timestamp. Used for automatic sleep onset estimation.

Actions Performed:

- Receives watch sensor data via WatchConnectivity

- Extracts audio and sonar features on-device (privacy preservation)

- Streams feature vectors to cloud server via WebSocket

- Relays cue commands from server to watch

- Performs morning HealthKit sync for calibration data

- Hosts user-facing dashboard, settings, and dream journal

## 4.3 Cloud Server

The server performs real-time sleep stage inference and manages the per-user calibration loop. It has access to full compute resources unconstrained by mobile hardware or battery.

Responsibilities:

- Receives real-time feature streams from iPhone via WebSocket

- Runs temporal sleep stage classifier (outputs probability distribution across Wake, Light, Deep, REM)

- Manages cue delivery logic (timing, intensity, escalation, abort conditions)

- Stores historical sleep data per user

- Performs nightly calibration against Apple’s HealthKit sleep stages

- Retrains per-user model periodically based on accumulated labeled data

- Correlates cue timing with dream journal lucidity reports to optimize cue strategy

# 5. Data Flow

## 5.1 Nighttime (Real-Time Loop)

The following sequence runs continuously from sleep onset detection until morning wake detection:

- Watch collects HR, HRV, and accelerometer data at configured sampling rates

- Watch sends summarized sensor packets to iPhone every 5–10 seconds via WatchConnectivity

- iPhone simultaneously runs sonar analysis and passive audio feature extraction

- iPhone combines watch data with phone sensor features into a unified feature vector

- iPhone streams feature vectors to cloud server via persistent WebSocket connection

- Server runs sleep stage classifier on rolling window of feature vectors

- If REM confidence exceeds threshold for sustained duration, server initiates cue protocol

- Server sends cue command (pattern, intensity) to iPhone

- iPhone relays cue command to watch via WatchConnectivity

- Watch delivers haptic cue and begins streaming post-cue sensor data at elevated rate

- Server monitors post-cue data for wake indicators (motion spike, HR spike). If detected, aborts further cues for this REM period

- If still in REM after cooldown period, server may escalate cue intensity and repeat

## 5.2 Morning (Calibration Sync)

Triggered when the system detects sustained waking or the user opens the app:

- iPhone queries HealthKit for Apple’s official sleep stage data with timestamps (REM, Core, Deep, Awake)

- iPhone uploads labeled sleep stage timeline to server

- Server aligns Apple’s labels with its own real-time predictions from the previous night

- Server computes accuracy metrics: REM precision, recall, onset latency, false positive rate

- Server updates per-user model weights based on prediction errors

- Server correlates cue delivery timestamps with Apple’s confirmed REM periods

- User completes dream journal entry (lucidity yes/no, cue awareness, dream narrative)

- Server incorporates journal data into cue strategy optimization

# 6. Sleep Stage Detection Model

## 6.1 Feature Set

Each feature vector represents a 30-second epoch and contains the following engineered features, computed from rolling windows of raw sensor data:

| Feature | Source | Window | REM Signature |
| --- | --- | --- | --- |
| Mean heart rate | Watch HR sensor | 5 min | Elevated (approaches waking levels) |
| HR standard deviation | Watch HR sensor | 5 min | High (irregular, variable) |
| HR rate of change | Watch HR sensor | 10 min | Rising trend at REM onset |
| HRV (RMSSD) | Watch HR sensor | 5 min | Shifted LF/HF ratio |
| Accelerometer energy | Watch accelerometer | 2 min | Very low (muscle atonia) |
| Micro-movement count | Watch accelerometer | 2 min | Occasional twitches possible |
| Breathing rate (sonar) | iPhone sonar | 3 min | Variable, irregular |
| Breathing regularity (CV) | iPhone sonar | 3 min | High coefficient of variation |
| Breathing depth (amplitude) | iPhone sonar | 3 min | Variable amplitude |
| Snore acoustic signature | iPhone mic | 5 min | Changed spectral properties |
| Body movement audio flag | iPhone mic | 1 min | Absent (stillness) |
| Time since sleep onset | Computed | N/A | Predictive of cycle position |
| Estimated cycle number | Computed | N/A | Later cycles = longer REM |
| Minutes since last transition | Computed | N/A | Temporal context |

## 6.2 Model Architecture

Phase 1 (Prototype): Gradient boosted classifier (XGBoost) with hand-engineered temporal features as described above. This approach is interpretable, fast to train, and works with limited initial data. Outputs a probability distribution across four classes: Wake, Light, Deep, REM.

Phase 2 (Production): LSTM or Transformer-based temporal model that operates on sequences of feature vectors (e.g., 60 consecutive 30-second epochs = 30-minute rolling window). This captures longer-range temporal dependencies like sleep cycle position and stage transition patterns that XGBoost cannot model natively.

Training Data: Initial model trained on public polysomnography datasets (MESA, SHHS, Sleep-EDF) using only cardiac and motion features (no EEG) as inputs with expert-scored sleep stages as labels. Fine-tuned per-user using Apple’s HealthKit sleep stage data as proxy ground truth.

## 6.3 Calibration Strategy

The nightly calibration loop is the system’s primary competitive advantage. Each morning:

- Apple’s sleep stage labels (trained against clinical polysomnography) serve as ground truth

- Server compares per-epoch predictions against Apple’s labels to compute accuracy metrics

- Wrist temperature trend data from HealthKit is correlated with REM timing across multiple nights to refine circadian-informed cycle predictions

- Per-user model is updated using accumulated labeled epochs (after 30 nights: ~30,000 labeled epochs per user)

- Cue delivery outcomes are correlated with confirmed REM status and dream journal reports to optimize timing, intensity, and frequency

Key Insight Apple invested years training their sleep stage classifier against clinical polysomnography data. Project Lullaby uses Apple’s classifier as a free nightly ground truth source, enabling continuous per-user model refinement without requiring any user to undergo a clinical sleep study.

# 7. Haptic Cue Delivery System

## 7.1 Cue Design

The haptic cue is a distinctive rhythmic vibration pattern delivered through the Apple Watch’s Taptic Engine. The pattern must be:

- Distinctive: Clearly different from notification haptics, alarms, or other watch feedback

- Learnable: Simple enough that users can memorize it and associate it with a “I am dreaming” intention during pre-sleep conditioning

- Scalable: Available at multiple intensity levels for adaptive escalation

- Non-waking: At its lowest intensity, gentle enough to be incorporated into the dream rather than triggering arousal

## 7.2 Delivery Protocol

- REM detected with confidence above threshold (default: 85%) for at least 3 consecutive minutes

- Wait for the cue window: 5–10 minutes into the REM period (dream is established but not at deepest REM)

- Deliver cue at Level 1 intensity (lowest haptic)

- Monitor watch sensors for 30–60 seconds post-cue

- If still in REM (no motion spike, no HR spike, breathing still irregular): wait cooldown period, then deliver Level 2 cue

- If wake indicators detected (motion spike, HR spike, breathing regularization): abort cues for this REM period

- Maximum cues per REM period: configurable, default 3

- Maximum cues per night: configurable, default 6

## 7.3 Pre-Sleep Conditioning

Before each sleep session, the app guides the user through a brief conditioning exercise:

- The watch plays the haptic pattern at full intensity so the user can clearly feel it

- The app prompts the user to close their eyes, feel the pattern, and mentally repeat: “When I feel this, I am dreaming”

- Pattern repeats 3–5 times with guided breathing between repetitions

- Exercise takes approximately 2 minutes

This conditioning builds the association between the haptic pattern and the concept of dreaming. Over time, the dreaming mind learns to recognize the pattern as an anomaly that triggers lucid awareness.

# 8. User Experience Flow

## 8.1 First-Time Setup

- User downloads iOS app and watchOS companion

- App requests permissions: HealthKit (read sleep stages, HR, HRV, respiratory rate, wrist temperature, SpO2; write sleep analysis), Microphone, Motion & Fitness

- User creates account (for cloud sync and per-user model)

- Onboarding tutorial explains the system, sets expectations (calibration period before cueing begins), and introduces pre-sleep conditioning

- User sets preferred sleep schedule or enables auto-detection

## 8.2 Nightly Flow

- Evening: user charges watch to sufficient level (app reminds if below 40%)

- Bedtime: user opens app, completes 2-minute pre-sleep conditioning exercise

- User places phone on nightstand and goes to bed

- App detects sleep intent (phone on charger + screen off + no interaction) and activates monitoring

- All night: system monitors silently. Watch collects biometrics, phone runs sonar and audio analysis, server classifies sleep stages and delivers cues when appropriate

- Morning: system detects waking. Phone syncs Apple’s sleep stage data. App presents sleep report

- User completes dream journal entry

## 8.3 Morning Dashboard

The morning report provides:

- Sleep stage timeline visualization (the system’s real-time classification alongside Apple’s official classification for transparency)

- Total sleep time, time in each stage, sleep efficiency

- Breathing pattern analysis (rate, regularity, any flagged anomalies)

- Cue delivery log: when cues were sent, intensity, whether they appeared to be incorporated or caused arousal

- Model confidence score: how accurately the system predicted sleep stages for this night

- Multi-night trends: sleep quality, REM duration, lucidity success rate over time

# 9. Development Phases

## Phase 1: Data Collection Prototype

Goal: Validate that REM can be reliably distinguished from other sleep stages using available sensor data.

Duration: 2–3 weeks

Deliverable: A minimal watchOS + iOS app that collects and logs all sensor data during sleep, plus a morning sync that pulls Apple’s sleep stages. Raw data exported for offline analysis.

Scope:

- watchOS app: start HKWorkoutSession, log HR, HRV, accelerometer to local file

- iOS app: run 19kHz sonar tone + mic recording, extract breathing features, log to file

- iOS app: passive audio recording with on-device feature extraction (breathing rate, snore detection)

- Morning sync: pull HKCategoryValueSleepAnalysis from HealthKit with stage timestamps

- Data export: structured JSON/CSV output for each night

- No server. No ML model. No cues. Just raw data collection.

Success Criteria: Visual inspection of plotted data shows distinguishable patterns between Apple-labeled REM epochs and non-REM epochs across heart rate, breathing regularity, and motion features.

## Phase 2: Offline Classification

Goal: Train and validate a sleep stage classifier on collected data.

Duration: 2–4 weeks

Deliverable: Trained XGBoost model with measured accuracy metrics against Apple’s labels.

Scope:

- Engineer features from raw sensor data (as defined in Section 6.1)

- Train XGBoost classifier using Apple’s sleep stages as labels

- Evaluate: per-epoch accuracy, REM precision/recall, confusion matrix

- Establish baseline: if REM F1-score is below 0.6, investigate which signals are weak and whether hardware/placement changes improve them

- Iterate on feature engineering based on error analysis

Success Criteria: REM detection F1-score of 0.65 or higher. REM onset detection within 3 minutes of Apple’s labeled onset for 70%+ of REM periods.

## Phase 3: Real-Time Server Pipeline

Goal: Move classification from offline to real-time with server-based inference.

Duration: 3–4 weeks

Deliverable: WebSocket-connected server that receives live feature streams and returns sleep stage predictions in real-time.

Scope:

- Server application (FastAPI or equivalent) with WebSocket endpoint

- Real-time feature vector ingestion and classification

- Persistent storage for per-user historical data

- Morning calibration endpoint: receives Apple’s labels, computes accuracy, stores for retraining

- iPhone app upgraded to stream features to server instead of logging locally

- Basic admin dashboard for monitoring system performance

Success Criteria: End-to-end latency from sensor event to server classification under 5 seconds. Classification accuracy matches or exceeds offline model.

## Phase 4: Haptic Cue Delivery

Goal: Implement the full cue delivery loop.

Duration: 2–3 weeks

Deliverable: Complete system that detects REM and delivers haptic cues through the Apple Watch.

Scope:

- Server-side cue decision logic (timing, confidence threshold, escalation, abort)

- Command relay: server → iPhone → watch

- watchOS haptic pattern implementation with configurable intensity levels

- Post-cue monitoring and wake detection

- Pre-sleep conditioning flow in iOS app

- Dream journal interface

Success Criteria: Cues delivered during confirmed REM periods (verified next morning) at least 80% of the time. User-reported sleep disruption rate below 15% of cue events.

## Phase 5: Per-User Calibration & Optimization

Goal: Implement the self-improving calibration loop.

Duration: 3–4 weeks

Deliverable: System that measurably improves REM detection accuracy and cue effectiveness over time for each individual user.

Scope:

- Automated nightly model retraining pipeline

- Per-user model storage and versioning

- Wrist temperature trend integration for circadian cycle prediction

- Cue strategy optimization based on lucidity reports

- A/B testing framework for cue parameters (intensity, timing, pattern)

- User-facing accuracy transparency (show prediction vs. Apple comparison)

Success Criteria: Measurable improvement in REM detection accuracy after 14 nights of use compared to first 3 nights. User-reported lucid dream frequency higher than baseline (pre-app) self-reports.

## Phase 6: Polish & Launch

Goal: Production-ready app for public release.

Duration: 4–6 weeks

Scope:

- Full UI/UX design and implementation

- Onboarding flow with tutorial content

- Battery optimization and power management

- Privacy policy, data handling documentation

- App Store review preparation (health-related app guidelines)

- Beta testing with lucid dreaming community

- Marketing site and launch strategy

# 10. Privacy and Safety

## 10.1 Data Privacy

- Raw audio never leaves the device. All audio processing (breathing analysis, snore detection, sonar) happens on-device. Only numeric feature vectors are transmitted to the server.

- Sonar data is inherently non-sensitive. It consists of amplitude and phase measurements of an ultrasonic tone, not recordable audio content.

- Biometric data is encrypted in transit and at rest. All server communication uses TLS. Stored data is encrypted per-user.

- User controls their data. Full data export and account deletion available at any time.

- Optional offline mode. A lighter on-device-only model for users who prefer no cloud processing.

## 10.2 Sleep Safety

- Conservative cue thresholds to minimize sleep disruption

- Automatic abort on any sign of user arousal

- Maximum cue limits per REM period and per night

- First-week observation period with zero cues

- User can disable cueing at any time while retaining sleep tracking

- Clear disclaimer: this is a wellness tool, not a medical device

# 11. Technical Requirements

## 11.1 Hardware Requirements

- Apple Watch Series 4 or later (required for HealthKit sleep stage data)

- iPhone running iOS 16 or later

- Apple Watch charged to at least 30% at bedtime

- iPhone placed on nightstand within 1 meter of bed, connected to charger

## 11.2 Software Stack

| Component | Technology | Notes |
| --- | --- | --- |
| watchOS App | Swift, SwiftUI, HealthKit, CoreMotion, WatchConnectivity | Minimum deployment: watchOS 9 |
| iOS App | Swift, SwiftUI, HealthKit, AVFoundation, Accelerate, CoreML | Minimum deployment: iOS 16 |
| Server | Python, FastAPI, WebSocket, PostgreSQL | Alternative: Node.js with equivalent stack |
| ML Pipeline | Python, XGBoost (Phase 1–2), PyTorch (Phase 3+) | Model served via ONNX or CoreML export |
| Infrastructure | Cloud hosting with WebSocket support | Low-latency requirement (<5 sec round trip) |

## 11.3 Key API Frameworks

| Framework | Purpose | Key Classes/Methods |
| --- | --- | --- |
| HealthKit | Sleep stages, HR, HRV, temperature, SpO2 | HKWorkoutSession, HKAnchoredObjectQuery, HKCategoryValueSleepAnalysis |
| CoreMotion | Accelerometer, gyroscope, device motion | CMMotionManager, CMSensorRecorder, CMBatchedSensorManager |
| WatchConnectivity | Watch ↔ Phone communication | WCSession, transferUserInfo, sendMessage |
| AVFoundation | Sonar tone generation + mic recording | AVAudioEngine, AVAudioPlayerNode, AVAudioInputNode |
| Accelerate | Signal processing (FFT, filtering) | vDSP framework for sonar signal analysis |

# 12. Open Questions

The following questions require empirical answers from prototype testing and cannot be resolved through design alone:

- Sonar range and reliability: At what maximum distance from the bed can the iPhone sonar reliably detect breathing? How do thick blankets, different mattress types, and user sleeping positions affect signal quality?

- Partner interference: Can sonar isolate one person’s breathing from a bed partner’s? If not, how much does this degrade classification accuracy, and can watch-only features compensate?

- Battery impact: What is the actual battery drain on the Apple Watch running an HKWorkoutSession all night? Is the drain acceptable for daily use?

- WatchConnectivity latency: What is the real-world latency of the cue command relay path (server → phone → watch)? Is it consistent enough for time-sensitive cue delivery?

- Haptic perception threshold during REM: What is the minimum Apple Watch haptic intensity that a sleeping user in REM can perceive without waking? This may vary significantly between individuals.

- App Store approval: Will Apple approve an app that intentionally delivers haptic cues during sleep? Are there health-related app guidelines that require clinical validation?

- Background execution limits: Can the iOS app maintain continuous sonar + audio processing and WebSocket connection reliably throughout an 8-hour sleep session without being terminated by the OS?

# 13. Competitive Landscape

| Product | Approach | REM Detection | Cue Method | Status |
| --- | --- | --- | --- | --- |
| Remee | LED sleep mask | None (timer only) | Red LEDs | Available, poor results |
| Neuroon | EEG headband | EEG-based | LEDs + vibration | Discontinued |
| iBand+ | EEG headband | EEG-based | LEDs + audio | Failed Kickstarter delivery |
| Lucid Dreamer | EEG headband | EEG-based | Audio cues | Limited availability |
| Sleep as Android | Phone app | Sonar/accelerometer | Audio cues | Available, no watch integration |
| Project Lullaby | Watch + Phone + Cloud | Multi-sensor fusion + ML | Haptic (wrist) | In development |

Competitive Advantage Project Lullaby is the only system that: (1) requires zero additional hardware, (2) uses multi-sensor fusion across two devices, (3) incorporates ultrasonic sonar for contactless breathing analysis, (4) calibrates against Apple’s clinical-grade sleep staging nightly, and (5) builds a personalized per-user model that improves over time.

# 14. Future Considerations: AirPods Integration

Apple’s AirPods Pro 3 (released September 2025) introduced in-ear heart rate monitoring via optical PPG sensors that pulse LED light 256 times per second. While the current API is limited to workout-context heart rate only, Apple’s trajectory strongly suggests expanded passive health sensing from earbuds in the near future. This section outlines how AirPods could become a third sensor node in Project Lullaby’s architecture.

## 14.1 Current AirPods Pro 3 Capabilities

| Capability | Developer Access | Limitation |
| --- | --- | --- |
| Heart rate (optical PPG) | Yes, via HealthKit during workouts | Workout-context only. No passive or continuous monitoring. |
| Heart rate variability | No | No HRV samples generated. No SDNN or R-R intervals exposed. |
| Accelerometer / Gyroscope | Limited (spatial audio head tracking) | Not exposed as raw data for third-party health analysis. |
| Temperature sensor | Not yet available | Rumored but not shipped as developer-accessible feature. |
| Hearing health | Yes (test, aid, protection) | Not relevant to sleep staging. |
| In-ear microphone | No raw access | Used for ANC and transparency only. |

## 14.2 Why the Ear Canal Matters for Sleep

The ear canal is a physiologically superior measurement site compared to the wrist for several reasons relevant to sleep monitoring:

- Heart rate accuracy: The ear canal provides consistent sensor-to-skin contact and predictable blood flow through the superficial temporal artery. Unlike the wrist, there is minimal motion artifact during sleep because the head moves far less than the arm.

- Core body temperature: The ear canal sits closer to the tympanic membrane, which reflects core body temperature more accurately than wrist skin temperature. Core temperature follows a tighter circadian rhythm and could provide a stronger predictive signal for sleep cycle timing.

- In-ear audio: An in-ear microphone could detect breathing patterns via bone conduction and internal body sounds, eliminating ambient noise problems that limit the iPhone’s bedside microphone. This would provide a cleaner respiratory signal independent of room acoustics, blankets, or partner noise.

- EEG-adjacent potential: In-ear EEG is an active research area. Sensors in the ear canal can detect brain electrical activity — the gold standard for sleep staging. If Apple ever adds EEG capability to AirPods, it would fundamentally transform sleep stage detection accuracy.

## 14.3 Projected Integration Path

Based on Apple’s trajectory of expanding health features across devices, the following integration milestones are anticipated:

Near-term (1–2 years):

- Apple enables passive heart rate monitoring from AirPods beyond workout context

- Temperature sensor becomes developer-accessible via HealthKit

- Lullaby integration: Fuse AirPods HR with Watch HR for higher-confidence cardiac signal. Use ear temperature as a real-time circadian phase indicator to improve cycle prediction.

Mid-term (2–3 years):

- Apple exposes HRV and R-R interval data from AirPods

- In-ear respiratory rate estimation via internal microphone becomes available

- Lullaby integration: Triple-source sensor fusion (watch cardiac + phone sonar/audio + AirPods cardiac/respiratory/temperature). Dramatically reduces reliance on any single sensor.

Long-term (3–5 years):

- Sleep-optimized AirPods form factor (smaller, comfortable for side-sleepers)

- Potential in-ear EEG sensing for direct brain-state measurement

- AirPods as cue delivery device: audio tones played into the ear canal during REM, calibrated to be dream-incorporated rather than waking

- Lullaby integration: Near-clinical-grade sleep staging from consumer hardware. Personalized cue modality choice (haptic via watch, audio via AirPods, or combined).

## 14.4 Design Considerations

- Comfort: Current AirPods are not designed for all-night wear. Many users find in-ear devices uncomfortable during sleep, especially side-sleepers. AirPods should be an optional enhancement, never a requirement.

- Battery: AirPods Pro 3 provide approximately 6.5 hours with heart rate sensing active. An 8-hour sleep session would require mid-sleep charging or future battery improvements.

- Architecture: The sensor fusion model should gracefully handle variable input sources — watch-only, watch+phone, or watch+phone+AirPods — weighting available inputs accordingly. AirPods data is additive, not required.

- Audio cue delivery: If AirPods are worn during sleep, they enable audio-based lucid dream cues (spoken phrases, tones, binaural beats) delivered directly to the ear. LaBerge’s research used audio cues successfully. This could complement or replace haptic wrist cues for some users.

Strategic Implication AirPods integration is not required for Project Lullaby’s initial launch. However, the architecture should be designed with extensibility in mind. The sensor fusion model, server inference pipeline, and cue delivery system should all support pluggable input and output sources so that AirPods — or any future Apple health peripheral — can be integrated without rearchitecting the system.

# 15. Success Metrics

| Metric | Target | Measurement |
| --- | --- | --- |
| REM detection precision | ≥80% | Percentage of predicted REM epochs confirmed by Apple’s data |
| REM detection recall | ≥70% | Percentage of Apple’s REM epochs caught by the model |
| REM onset latency | <3 minutes | Time from Apple’s labeled REM onset to model’s detection |
| Cue-during-REM rate | ≥80% | Percentage of cues delivered during confirmed REM |
| Sleep disruption rate | <15% | Percentage of cues that appear to cause arousal |
| User-reported lucidity rate | >20% | Nights with lucid dream reports (after calibration period) |
| Nightly accuracy improvement | Measurable | Statistically significant improvement after 14 nights vs. first 3 |
| Retention (30-day) | >60% | Users still active after 30 days |

# 16. Immediate Next Steps

- Set up development environment: Install Xcode, create project with iOS and watchOS targets, configure HealthKit and microphone entitlements.

- Build Phase 1 data logger: Minimal app that logs HR, HRV, accelerometer (watch), sonar and audio features (phone), and Apple sleep stages (morning sync).

- Collect personal sleep data: Sleep with the data logger for 5–7 consecutive nights to build initial dataset.

- Analyze and visualize: Plot sensor data against Apple’s sleep stage labels. Determine if REM is visually distinguishable in the data.

- Decide go/no-go: If signal separation is clear, proceed to Phase 2. If not, investigate sensor improvements before continuing.

End of Document